BOM – PCB A

| QTY | NAME | DESCRIPTION | SEL |
| --- | --- | --- | --- |
| 6 | STACKPOLE RMCF0805ZT0R00TR-ND | RESISTOR 0 OHM | SEL.000028 (4400 PZ) |
| 3 | YAGEO 311-100CRTR-ND (RC0805FR-07100RL) | RESISTOR 100 OHM | SEL.000034 (4400 PZ) |
| 1 | STACKPOLE RMCF0805FT150RTR-ND | RESISTOR 150 OHM | SEL.000038 (4400 PZ) |
| 1 | YAGEO 311-220FRTR-ND (RC1206FR-07220RL) | RESISTOR 220 OHM | SEL.001229 (20 PZ) |
| 3 | STACKPOLE RMCF0805FT1K00 | RESISTOR 1 KOHM | SEL.000049 (4400 PZ) |
| 9 | YAGEO 311-10KARTR-ND (RC0805JR-0710KL) | RESISTOR 10 KOHM | SEL.000062 (4400 PZ) |
| 2 | Kemet C0805C100J5GACTU | CAPACITOR 10 pF | SEL.000911 (984 PZ) |
| 1 | Kemet C0805C101J5GACTU | CAPACITOR 100 pF | SEL.000857 (990 PZ) |
| 19 | Kemet C0805C104K3RAC7800 | CAPACITOR 100 nF | SEL.000087 (1000 PZ) |
| 4 | Samsung CL21B475KOFNNNE | CAPACITOR 4.7 uF | SEL.001701 (4 PZ) |
| 1 | Yageo CC0805KKX7R7BB105 | CAPACITOR 1 uF | SEL.000086 (1000 PZ) |
| 5 | Samsung CL21B106KOQNNNE | CAPACITOR 10 uF | SEL.000690 (29 PZ) |
| 2 | Kyocera AVX F381A106MMA | CAPACITOR 10 uF | SEL.001502 (6 PZ) |
| 1 | Kyocera AVX F951A226MPAAQ2 | CAPACITOR 22 uF | SEL.000755 (10 PZ) |
| 2 | BAS40-7-F | DIODE | SEL.000247 (72 PZ) |
| 1 | ONSEMI MBRA340T3G | DIODE SCHOTTKY 40V 3A SMA | SEL.001040 (16 PZ) |
| 1 | ONSEMI BC846BLT1G | TRANS NPN 65V 0.1A SOT23-3 | SEL.001790 (21 PZ) |
| 1 | DIOTEC BC857B-AQ | TRANS PNP 45V 0.1A SOT23-3 | SEL.000739 (8 PZ) |
| 2 | MMZ1608B601CTAH0 | INDUCTOR; SMT (0603); FERRITE-BEAD; 600; TOL=+/-25%; 0.5A; -55 DEGC TO +125 DEGC | SEL.001136 (163 PZ) |
| 1 | NCV1117ST33T3G | Linear Voltage Regulator IC Positive Fixed 1 Output 1A SOT-223 | SEL.000321 (5 PZ) |
| 1 | NCV1117STAT3G | IC REG LINEAR POS ADJ 1A SOT-223 | NOT AVAILABLE |
| 1 | CG04874-8M | 8 MHz ±20ppm Crystal 8pF 500 Ohms 2-SMD, No Lead | SEL.000471 (1 PZ) |
| 1 | MCP6001UT-E/OT | Microchip MCP6001UT-E/OT Op Amp, 1MHz Rail-Rail, 1.8 - 6 V, 5-Pin SOT-23 ¦ Microchip Technology Inc. MCP6001UT-E/OT | SEL.000435 (10 PZ) |
| 1 | STM32F446RCT6TR | ARM® Cortex®-M4 STM32F4 Microcontroller IC 32-Bit Single-Core 180MHz 256KB (256K x 8) FLASH 64-LQFP (10x10) | SEL.001604 (1 PZ) |
| 1 | LED-LTST-C190KRKT | Red 631nm LED Indication - Discrete 2V 0603 (1608 Metric) | SEL.000817 (254 PZ) |
| 2 | OMRON SW1020TR-ND | Switch Tactile N.O. SPST Round Button Gull Wing 0.05A 12VDC 1.5N SMD Embossed T/R | SEL.000613 (21 PZ) |
| 1 | MCP9808T-E/MC | TEMPERATURE SENSOR | - |
| 1 | TCD1304DG8ZAW | LINEAR CCD DETECTOR | - |
| 1 | AMPHENOL ICC (FCI)10118194-0001LF | Micro USB, Input Output Connectors, B TYPE RECEPTACLE with flange | SEL.000758 (47 PZ) |
| 1 | USBLC6-4SC6 | VERY LOW CAPACITANCE ESD PROTECTION | SEL.000267 (25 PZ) |
| 1 | SILICON LABS 336-1160-5-ND (CP2102-GM) | USB Bridge, USB to UART USB 2.0 UART Interface 28-QFN (5x5) | SEL.000346 (6 PZ) |
| 3 | Generic 40 mils drill size, 100 mils step. TOTAL: 36 PIN | Pin header x3 |  |
| 1 |  | Pin header x4 |  |
| 1 |  | Pin header x5 |  |
| 3 |  | Pin header x6 |  |
| 1 | HARWIN M20-7861546 | PIN SOCKET SMD 2x15 (CCD SENSOR) | NOT AVAILABLE |
| 1 | PHOENIX 1706207 | TERM BLOCK HDR 2POS 2.54MM | NOT AVAILABLE |
| 1 | PHOENIX 1821465 | TERM BLOCK HDR 9POS 2.54MM | NOT AVAILABLE |

BOM – PCB B

| QTY | NAME | DESCRIPTION | SEL |
| --- | --- | --- | --- |
| 7 | STACKPOLE RMCF0805ZT0R00TR-ND | RESISTOR 0 OHM | SEL.000028 (4400 PZ) |
| 1 | STACKPOLE RMCF0805FT10R0TR-ND | RESISTOR 10 OHM | SEL.000031 (4400 PZ) |
| 6 | YAGEO 311-10KARTR-ND (RC0805JR-0710KL) | RESISTOR 10 KOHM | SEL.000062 (4400 PZ) |
| 1 | Kemet C0805C101J5GACTU | CAPACITOR 100 pF | SEL.000857 (990 PZ) |
| 7 | Kemet C0805C104K3RAC7800 | CAPACITOR 100 nF | SEL.000087 (1000 PZ) |
| 1 | Taiyo Yuden 587-2994-2-ND (UMK316AB7475KL-T) | CAPACITOR 4.7 uF | SEL.000799 (500 PZ) |
| 1 | Samsung 1276-1075-2-ND (CL31A106KAHNNNE) | CAPACITOR 10 uF | SEL.001145 (990 PZ) |
| 4 | Panasonic PCE3836TR-ND (EEE-FK1V101XP) | CAPACITOR 100 uF | SEL.001144 (48 PZ) |
| 1 | ONSEMI FSV15150V | DIODE SCHOTTKY 150V 15A TO2773 | SEL.000715 (6 PZ) |
| 2 | AO 785-1106-2-ND (AOD417) | P-Channel 30V 25A, 2.5W Ta, 50W Tc Surface Mount TO-252, D-Pak | SEL.001625 (8 PZ) |
| 1 | MICROCHIP TC4420COA713TR-ND | MOSFET Power Driver 6A,4.5 to 18V, Non-Inverting,8-Pin SOIC ¦ Microchip Technology Inc. TC4420COA | SEL.001374 (1 PZ) |
| 1 | WÜRTH 732-11711-ND (74437529203220) | FIXED IND 22UH 23.9A 4.9MOHM SMD | SEL.001103 (3 PZ) |
| 2 | POLOLU A4988 | Stepper Motor Driver Carrier | - |
| 1 | PHOENIX 1706207 | TERM BLOCK HDR 2POS 2.54MM | NOT AVAILABLE |
| 1 | PHOENIX 1821465 | TERM BLOCK HDR 9POS 2.54MM | NOT AVAILABLE |
| 2 | MOLEX WM11252-ND (436500402) | Connector Header Through Hole, Right Angle 4 position 0.118 (3.00mm) | SEL.001696 (3 PZ) |
| 4/2 | PIN SOCKET | 1 x 8 POS / 2 x 8 POS |  |